1. pickup free request from bddesc.freefst

if ( bddesc.freefst == 0 )

wait;    /* wait for more free request */

/* lockup TAS bit, then pick up request */

locktas ();

q= bddesc.freefst + IOP_BASE;

2. setup next free request in board descriptor

bddesc.freefst = q->q_next;

3. zero the q_next in q

q->q_next = 0;

4. setup data in request q

q->q_devtype = xx;

q->q_devnum = xx;

q->q_cmd = xx;

q->q_devun.block = xx;

q->q_mem = xx;

q->q_count = xx;

q->q_mmu = 0;

q->q_flag = 0;

5. put q at either bddesc.reqfst or bddesc.reqlst

if (bddesc.reqfst == 0)

bddesc.reqfst = bddesc.reqlst = q & 0xffff;

else {

lastq = bddesc.reqlst + IOP_BASE;

lastq->q_next = q & 0xffff;

q->q_prev = lastq & 0xffff;

bddesc.reqlst = q & 0xffff;

}

6. unlock TAS bit

unlocktas ();

7.  need to interrupt HSDT to take request

intrp (rp, DTREQ);		/* DTREQ=0 */

8. wait for HSDT to interrupt

lock TAS bit, then pickup q from bddesc.rsfst

locktas ();

q = bddesc.rsfst + IOP_BASE;

9. take return code

retcode1 = q->rc1; retcode2 = q->rc2;

10. take q off the respone queue

bddesc.rsfst = q->q_next;

if ( q->q_next == 0 )

bddesc.rslst = 0;

11. zero the queue and put back at the beginning of free list

zero (q);

q->q_next = bddesc.freefst;

bddesc.freefst = q & 0xffff;

12. unlock TAS bit

unlocktas();