/*

* below is a programming example for each of the possible iocp transactions

* using the above devq structure.

* assume ptr = &devq;

* read the disk directly into main memory

* peripheral <---> mainmem (assume default external dtb device is dma channel)

ptr->q_devtype = DTDISK;

ptr->q_cmd = PREAD;

ptr->q_devnum = phdrive;

ptr->q_count = bytecount;

ptr->q_devun.block = blkno;

ptr->q_extdtb = XDDFLT; /* use default external dtb device */

ptr->q_mem = main_mem;

ptr->q_mmu = (struct mmu *)0; /* contiguous main mem */

ptr->priority = 0;

* read the disk into dtiocp local memory

* peripheral <---> iocpmem

ptr->q_devtype = DTDISK;

ptr->q_cmd = PREAD;

ptr->q_devnum = phdrive;

ptr->q_count = bytecount;

ptr->q_devun.block = blkno;

ptr->q_extdtb = NOXDTB; /* no external dtb device */

ptr->q_mem = iocp_mem; /* free area in dtiocp local memory */

ptr->q_mmu = (struct mmu *)0; /* this field ignored */

ptr->priority = 0;

* transfer block from iocp memory to main memory

* iocpmem <---> mainmem

ptr->q_devtype = DTLBUF; /* iocp local buffer */

ptr->q_cmd = READ;

ptr->q_devnum = 0; /* this field ignored */

ptr->q_count = bytecount;

ptr->q_devun.lbuf = iocp_mem;

ptr->q_extdtb = XDTYPDMA | x; /* x == the dma's dtb id */

ptr->q_mem = main_mem;

ptr->q_mmu = (struct mmu *)0; /* contiguous main mem */

ptr->priority = 0;

* transfer data from an iocp peripheral to another iocp's local memory.

* this transaction requires special synchronization by the master cpu.

* two iocp boards must be programmed in the right sequence for this to

* work correctly.  in the programming examples below notice that the

* destination (iocp local memory) is programmed first.  when it is

* ready a signal is sent to the master cpu to program the other iocp

* card involved.

* assume	unsigned char dtb_master_id = 4; /* dtb id of the iocp peripheral */

* 			unsigned char dtb_slave_id = 5; /* dtb id of the destination iocp */

* iocp (dtb slave) <---> external iocp peripheral

ptr->q_devtype = DTLBUF;

ptr->q_cmd = WRITE;

ptr->q_devnum = 0; /* this field ignored */

ptr->q_count = bytecount;

ptr->q_devun.lbuf = iocp_mem;

ptr->q_extdtb = XDTYPMR | dtb_master_id; /* the external dtb device is master */

ptr->q_mem = 0; /* this field ignored */

ptr->q_mmu = (struct mmu *)0; /* this field ignored */

ptr->priority = 0;

* iocp peripheral (dtb master) <---> external iocp

ptr->q_devtype = DTDISK;

ptr->q_cmd = PREAD;

ptr->q_devnum = phdrive;

ptr->q_count = bytecount;

ptr->q_devun.block = blkno;

ptr->q_mem = 0; /* this field ignored */

ptr->q_extdtb = XDTYPSL | dtb_slave_id; /* the external dtb device is slave */

ptr->q_mmu = (struct mmu *)0; /* this field ignored */

ptr->priority = 0;