XCHECK 08032 ISE EXERCISER PROGRAM

USER GUIDE

INTRODUCTION

XCHECK is an exerciser program for 08032 ISE.  It is configured

as a command file processed by the idbg16 ISE debugger.  The

debugger, which runs on the host computer (VAX or SYS-16),

accepts as input the command files which XCHECK are composed

of, and translates their commands into actions on the ISE hard-

ware.  Revision 1.00 of idbg16 and revision V0009 of ISE firm-

ware are required for operation of XCHECK.

XCHECK is intended to provide diagnostic coverage beyond the

capabilities of the ISE on-board power-on diagnostic.  Wherever

possible, it is able to remove ambiguity as to which board or

assembly has failed.  XCHECK presents no ambiguous results;

the user is not required to guess whether a failure has been

detected.  XCHECK can be used at the delivery site to establish

functionality of an ISE unit upon receipt, and in the field

to identify ISE failure.  It is possible that XCHECK may detect

failures which are not detected by the power-on diagnostic.

XCHECK assumes the following elements are functional:

Power supplies -- +5V, -12V, +12V

Host computer -- VAX or SYS-16

Commumications -- ISE USARTs and all cables

ISE Emulation RAM

Pod to Box cables

16K chip set and firmware PROMs

XCHECK does not test the following elements:

Target interface -- cables, drivers, receivers

Slave interface contol logic

External (Status Pod) Inputs

GPIB interface

RS232 interfaces

OPERATION

In order to run XCHECK, the following steps must be performed

by the user:

1)  Disconnect ISE Target Cables from ISE, if attached.

2)  Reset ISE, using Master-Reset switch on ISE Pod.

3)  Invoke idbg16 on host.

4)  Enter a proper select-link command.

5)  Enter '@xcheck' to begin execution of XCHECK.

If a failure is detected by XCHECK, an error message will be

written to the console, execution of XCHECK will halt, and

the debugger will assume its normal command-processing state.

Prior to branching to the error-message file, XCHECK will write

the name of the command file in which the failure has been

detected.  Knowing the name of the command file in which the

failure occurred and the nature of the failure, the user can

refer to the "Program-Module Description" section of this document

to determine the location and name of the failing element.

The failing idbg16 "in" statement will be displayed on the

screen just prior to the error message generated, enabling the

user to determine what might be incorrect about a failing test.

GENERAL PROGRAM FLOW

XCHECK is composed of about 80 files.  These files are named

according to the ISE logic tested in each command file;  for

example, the file group 'adbrkx.ind' tests the ISE address/data

breakpointing system.  The following table details the order

in which the files of XCHECK are executed during an XCHECK run

in which no errors are detected.

1)  adtr1.ind --- test ISE Pod address mapper

2)  adtr2.ind

3)  adtr3.ind

4)  adtr4.ind

5)  adtr5.ind

6)  clkswt.ind -- tests ISE Pod clock-switching

7)  exbrka.ind -- tests ISE Board execution breakpoints

8)  exbrkb.ind

9)  exbrkc.ind

10)  adbrka.ind -- tests ISE Board addr/data breakpoints

11)  adbrkb.ind

12)  adbrkc.ind

13)  cbrk01.ind -- sliding-ones to breakpoint-compare rams

14)  cbrk02.ind

15)  cbrk04.ind

16)  cbrk08.ind

17)  cbrk10.ind

18)  cbrk20.ind

19)  cbrk40.ind

20)  cbrk80.ind

21)  cdst01.ind -- sliding-ones to break-distribution rams

22)  cdst02.ind

23)  cdst04.ind

24)  cdst08.ind

25)  cdst10.ind

26)  cdst20.ind

27)  cdst40.ind

28)  cdst80.ind

29)  trats1.ind -- tests memory-cycle trace

30)  trats2.ind

31)  trats3.ind

32)  trats4.ind

33)  trats5.ind

34)  trats6.ind -- tests program-flow trace

35)  trats7.ind

36)  imbrk1.ind -- sliding-ones to immediate-break rams

37)  imbrk2.ind

38)  imbrk4.ind

39)  extmrc.ind -- execution timer tests

40)  extmrm.ind

41)  extmri.ind

42)  extmrb.ind

43)  bkctrc.ind -- breakpoint counter tests

44)  bkctrm.ind

45)  bkctri.ind

46)  bkctrb.ind

PROGRAM-MODULE DESCRIPTION

This section describes each module of XCHECK with regard to the

ISE functions exercised and possible errors that might be

detected.

1)  adtr1.ind

Description:  This file maps Emulation RAM from all four

segments of the Address Translation logic, then tests the

mapped address space to verify that the mapping was properly

accomplished.  At the end of the test, segments B and C are

unmapped and B is tested to insure that the unmapping worked

properly.

Detectable failures:  If mapping or unmapping from a particular

segment should not function properly, XCHECK will write a

message of the format:

XCHECK HALTED DUE TO ERROR IN ADDRESS TRANSLATION SEGMENT "x"

where "x" is the name of the failed segment, and will pass

control to the idbg16 command handler.

2)  adtr2.ind

Description:  This file provides an second check of the

unmapping of segment B.

Detectable failures:  If the address space mapped by segment

B is found to be mapped, the message:

XCHECK HALTED DUE TO ERROR IN ADDRESS TRANSLATION SEGMENT "B"

will be written, and control will pass to the idbg16 command

handler.

3)  adtr3.ind

Description:  This file checks the unmapping of segment C.

Detectable failures:  If the address space mapped by segment

C is found to be mapped, the message:

XCHECK HALTED DUE TO ERROR IN ADDRESS TRANSLATION SEGMENT "C"

will be written, and control will pass to the idbg16 command

handler.

4)  adtr4.ind

Description:  This file provide a second check on the unmapping

of segment C.

Detectable failures:  If the address space mapped by segment

C is found to be mapped, the message:

XCHECK HALTED DUE TO ERROR IN ADDRESS TRANSLATION SEGMENT "C"

is written, and control is passed to the idbg16 command handler.

5)  adtr5.ind

Description:  This test checks that segments A and D have not

become unmapped during the unmapping of segments B and C, then

maps all of Emulation RAM into segment A and tests several data

patterns across the entire RAM.

Detectable failures:  If the address space mapped by segment

A is found to be not mapped, the message:

XCHECK HALTED DUE TO ERROR IN ADDRESS TRANSLATION SEGMENT "A"

is written, and control is passed to the idbg16 command handler.

If the address space mapped by segment D is found to be no

longer mapped, the message:

XCHECK HALTED DUE TO ERROR IN ADDRESS TRANSLATION SEGMENT "D"

is written, and control is passed to the idbg16 command handler.

6)  clkswt.ind

Description:  This file tests the ISE clock-switching logic by

specifying all possible clock settings;  and verifying ISE

operation by mapping memory and downloading and single-stepping

a program for each clock setting.

Detectable failures:  If the downloaded program is not executed

properly, the message:

XCHECK HALTED DUE TO IMPROPER BREAKPOINTING

OR EXECUTION FOLLOWING "STEP" COMMAND.

is written, and control is passed to the idbg16 command	handler.

7)  exbrka.ind

Description:  This file checks for proper operation of execution

breakpoint A by downloading a program into mapped memory, then

single-stepping through the breakpoint.

Detectable failures:  If an incorrect program counter value is

observed following one of the single-steps, the message:

XCHECK HALTED DUE TO IMPROPER BREAKPOINTING

OR EXECUTION FOLLOWING "STEP" COMMAND.

is written, and control is passed to the idbg16 command handler.

If a Compare Breakpoint Status code other than h'5f' is observed,

the message:

XCHECK HALTED DUE TO IMPROPER COMPARE-BREAKPOINT STATUS

is written, and control is passed to the idbg16 command handler.

8)  exbrkb.ind

Description:  This file checks for proper operation of execution

breakpoint B by downloading a program into mapped memory, then

single-stepping through the breakpoint.

Detectable failures:  If an incorrect program counter value is

observed following one of the single-steps, the message:

XCHECK HALTED DUE TO IMPROPER BREAKPOINTING

OR EXECUTION FOLLOWING "STEP" COMMAND.

is written, and control is passed to the idbg16 command handler.

If a Compare Breakpoint Status code other than h'77' is observed,

the message:

XCHECK HALTED DUE TO IMPROPER COMPARE-BREAKPOINT STATUS

is written, and control is passed to the idbg16 command handler.

9)  exbrkc.ind

Description:  This file checks for proper operation of execution

breakpoint C by downloading a program into mapped memory, then

single-stepping through the breakpoint.

Detectable failures:  If an incorrect program counter value is

observed following one of the single-steps, the message:

XCHECK HALTED DUE TO IMPROPER BREAKPOINTING

OR EXECUTION FOLLOWING "STEP" COMMAND.

is written, and control is passed to the idbg16 command handler.

If a Compare Breakpoint Status code other than h'7d' is observed,

the message:

XCHECK HALTED DUE TO IMPROPER COMPARE-BREAKPOINT STATUS

is written, and control is passed to the idbg16 command handler.

10)  adbrka.ind

Description:  This file checks the operation of address/data

breakpoint A by downloading a program into mapped memory and

executing it.  As many bits as are feasable are toggled between

two breakpoints that should be created during execution of this

file.

Detectable failures:  If an incorrect PC is observed following

an emulation, the message:

XCHECK HALTED DUE TO IMPROPER BREAKPOINTING

OR EXECUTION FOLLOWING "GO" COMMAND

is written, and control is passed to the idbg16 command handler.

If a Compare Breakpoint Status code other than h'5f' is observed,

the message:

XCHECK HALTED DUE TO IMPROPER COMPARE-BREAKPOINT STATUS

is written, and control is passed to the idbg16 command handler.

11)  adbrkb.ind

Description:  This file checks the operation of address/data

breakpoint B by downloading a program into mapped memory and

executing it.  As many bits as are feasable are toggled between

two breakpoints that should be created during execution of this

file.

Detectable failures:  If an incorrect PC is observed following

an emulation, the message:

XCHECK HALTED DUE TO IMPROPER BREAKPOINTING

OR EXECUTION FOLLOWING "GO" COMMAND

is written, and control is passed to the idbg16 command handler.

If a Compare Breakpoint Status code other than h'77' is observed,

the message:

XCHECK HALTED DUE TO IMPROPER COMPARE-BREAKPOINT STATUS

is written, and control is passed to the idbg16 command handler.

12)  adbrkc.ind

Description:  This file checks the operation of address/data

breakpoint C by downloading a program into mapped memory and

executing it.  As many bits as are feasable are toggled between

two breakpoints that should be created during execution of this

file.

Detectable failures:  If an incorrect PC is observed following

an emulation, the message:

XCHECK HALTED DUE TO IMPROPER BREAKPOINTING

OR EXECUTION FOLLOWING "GO" COMMAND

is written, and control is passed to the idbg16 command handler.

If a Compare Breakpoint Status code other than h'7d' is observed,

the message:

XCHECK HALTED DUE TO IMPROPER COMPARE-BREAKPOINT STATUS

is written, and control is passed to the idbg16 command handler.

13)  cbrk01.ind

Description:  This file checks breakpoint and trace for an input

pattern in which zeros are applied to all bits except bit 0 of

each of the Compare Breakpoint RAMs.  This is accomplished by

single-stepping an instruction at the address which satisfies

the bit 0 requirememt, and breakpointing and tracing the execution

of that instruction.

Detectable failures:  If the PC is incorrect following the

emulation, the message:

XCHECK HALTED DUE TO IMPROPER BREAKPOINTING

OR EXECUTION FOLLOWING "STEP" COMMAND.

is written, and control is passed to the idbg16 command handler.

If the Compare Breakpoint Status code is not h'15', the message:

XCHECK HALTED DUE TO IMPROPER COMPARE-BREAKPOINT STATUS

is written, and control is passed to the idbg16 command handler.

If the Trace Buffer Address Counter is not equal to one, the

message:

XCHECK HALTED DUE TO IMPROPER TRACE BUFFER-ADDRESS VALUE

is written, and control is passed to the idbg16 command handler.

If the content of Trace RAM 0 (low data) is not correct, the

message:

XCHECK HALTED DUE TO INCORRECT LOW-DATA TRACE

is written, and control is passed to the idbg16 command handler.

If the content of Trace RAM 1 (high data) is not correct, the

message:

XCHECK HALTED DUE TO INCORRECT HIGH-DATA TRACE

is written, and control is passed to the idbg16 command handler.

If the content of Trace RAM 2 (low address) is not correct, the

message:

XCHECK HALTED DUE TO INCORRECT LOW-ADDRESS TRACE

is written, and control is passed to the idbg16 command handler.

If the content of Trace RAM 3 (middle address) is not correct,

the message:

XCHECK HALTED DUE TO INCORRECT MIDDLE-ADDRESS TRACE

is written, and control is passed to the idbg16 command handler.

If the content of Trace RAM 4 (high address) is not correct, the

message:

XCHECK HALTED DUE TO INCORRECT HIGH-ADDRESS TRACE

is written, and control is passed to the idbg16 command handler.

If the content of Trace RAM 5 (status bits) is not correct, the

message:

XCHECK HALTED DUE TO INCORRECT STATUS-BIT TRACE

is written, and control is passed to the idbg16 command handler.

14)  cbrk02.ind

Description:  This file checks breakpoint and trace for an input

pattern in which zeros are applied to all bits except bit 1 of

each of the Compare Breakpoint RAMs.  This is accomplished by

single-stepping an instruction at the address which satisfies

the bit 1 requirement, and breakpointing and tracing the execution

of that instruction.

Detectable failures:  If the PC is incorrect following the

emulation, the message:

XCHECK HALTED DUE TO IMPROPER BREAKPOINTING

OR EXECUTION FOLLOWING "STEP" COMMAND.

is written, and control is passed to the idbg16 command handler.

If the Compare Breakpoint Status code is not zero, the message:

XCHECK HALTED DUE TO IMPROPER COMPARE-BREAKPOINT STATUS

is written, and control is passed to the idbg16 command handler.

If the Trace Buffer Address Counter is not equal to one, the

message:

XCHECK HALTED DUE TO IMPROPER TRACE BUFFER-ADDRESS VALUE

is written, and control is passed to the idbg16 command handler.

If the content of Trace RAM 0 (low data) is not correct, the

message:

XCHECK HALTED DUE TO INCORRECT LOW-DATA TRACE

is written, and control is passed to the idbg16 command handler.

If the content of Trace RAM 1 (high data) is not correct, the

message:

XCHECK HALTED DUE TO INCORRECT HIGH-DATA TRACE

is written, and control is passed to the idbg16 command handler.

If the content of Trace RAM 2 (low address) is not correct, the

message:

XCHECK HALTED DUE TO INCORRECT LOW-ADDRESS TRACE

is written, and control is passed to the idbg16 command handler.

If the content of Trace RAM 3 (middle address) is not correct,

the message:

XCHECK HALTED DUE TO INCORRECT MIDDLE-ADDRESS TRACE

is written, and control is passed to the idbg16 command handler.

If the content of Trace RAM 4 (high address) is not correct, the

message:

XCHECK HALTED DUE TO INCORRECT HIGH-ADDRESS TRACE

is written, and control is passed to the idbg16 command handler.

If the content of Trace RAM 5 (status bits) is not correct, the

message:

XCHECK HALTED DUE TO INCORRECT STATUS-BIT TRACE

is written, and control is passed to the idbg16 command handler.

15)  cbrk04.ind

Description:  This file checks breakpoint and trace for an input

pattern in which zeros are applied to all bits except bit 2 of

each of the Compare Breakpoint RAMs.  This is accomplished by

single-stepping an instruction at the address which satisfies

the bit 2 requirement, and breakpointing and tracing the execution

of that instruction.

Detectable failures:  If the PC is incorrect following the

emulation, the message:

XCHECK HALTED DUE TO IMPROPER BREAKPOINTING

OR EXECUTION FOLLOWING "STEP" COMMAND.

is written, and control is passed to the idbg16 command handler.

If the Compare Breakpoint Status code is not h'15', the message:

XCHECK HALTED DUE TO IMPROPER COMPARE-BREAKPOINT STATUS

is written, and control is passed to the idbg16 command handler.

If the Trace Buffer Address Counter is not equal to one, the

message:

XCHECK HALTED DUE TO IMPROPER TRACE BUFFER-ADDRESS VALUE

is written, and control is passed to the idbg16 command handler.

If the content of Trace RAM 0 (low data) is not correct, the

message:

XCHECK HALTED DUE TO INCORRECT LOW-DATA TRACE

is written, and control is passed to the idbg16 command handler.

If the content of Trace RAM 1 (high data) is not correct, the

message:

XCHECK HALTED DUE TO INCORRECT HIGH-DATA TRACE

is written, and control is passed to the idbg16 command handler.

If the content of Trace RAM 2 (low address) is not correct, the

message:

XCHECK HALTED DUE TO INCORRECT LOW-ADDRESS TRACE

is written, and control is passed to the idbg16 command handler.

If the content of Trace RAM 3 (middle address) is not correct,

the message:

XCHECK HALTED DUE TO INCORRECT MIDDLE-ADDRESS TRACE

is written, and control is passed to the idbg16 command handler.

If the content of Trace RAM 4 (high address) is not correct, the

message:

XCHECK HALTED DUE TO INCORRECT HIGH-ADDRESS TRACE

is written, and control is passed to the idbg16 command handler.

If the content of Trace RAM 5 (status bits) is not correct, the

message:

XCHECK HALTED DUE TO INCORRECT STATUS-BIT TRACE

is written, and control is passed to the idbg16 command handler.

16)  cbrk08.ind

Description:  This file checks breakpoint and trace for an input

pattern in which zeros are applied to all bits except bit 3 of

each of the Compare Breakpoint RAMs.  This is accomplished by

single-stepping an instruction at the address which satisfies

the bit 3 requirement, and breakpointing and tracing the execution

of that instruction.

Detectable failures:  If the PC is incorrect following the

emulation, the message:

XCHECK HALTED DUE TO IMPROPER BREAKPOINTING

OR EXECUTION FOLLOWING "STEP" COMMAND.

is written, and control is passed to the idbg16 command handler.

If the Compare Breakpoint Status code is not h'15', the message:

XCHECK HALTED DUE TO IMPROPER COMPARE-BREAKPOINT STATUS

is written, and control is passed to the idbg16 command handler.

If the Trace Buffer Address Counter is not equal to one, the

message:

XCHECK HALTED DUE TO IMPROPER TRACE BUFFER-ADDRESS VALUE

is written, and control is passed to the idbg16 command handler.

If the content of Trace RAM 0 (low data) is not correct, the

message:

XCHECK HALTED DUE TO INCORRECT LOW-DATA TRACE

is written, and control is passed to the idbg16 command handler.

If the content of Trace RAM 1 (high data) is not correct, the

message:

XCHECK HALTED DUE TO INCORRECT HIGH-DATA TRACE

is written, and control is passed to the idbg16 command handler.

If the content of Trace RAM 2 (low address) is not correct, the

message:

XCHECK HALTED DUE TO INCORRECT LOW-ADDRESS TRACE

is written, and control is passed to the idbg16 command handler.

If the content of Trace RAM 3 (middle address) is not correct,

the message:

XCHECK HALTED DUE TO INCORRECT MIDDLE-ADDRESS TRACE

is written, and control is passed to the idbg16 command handler.

If the content of Trace RAM 4 (high address) is not correct, the

message:

XCHECK HALTED DUE TO INCORRECT HIGH-ADDRESS TRACE

is written, and control is passed to the idbg16 command handler.

If the content of Trace RAM 5 (status bits) is not correct, the

message:

XCHECK HALTED DUE TO INCORRECT STATUS-BIT TRACE

is written, and control is passed to the idbg16 command handler.

17)  cbrk10.ind

Description:  This file checks breakpoint and trace for an input

pattern in which zeros are applied to all bits except bit 4 of

each of the Compare Breakpoint RAMs.  This is accomplished by

single-stepping an instruction at the address which satisfies

the bit 4 requirement, and breakpointing and tracing the execution

of that instruction.

Detectable failures:  If the PC is incorrect following the

emulation, the message:

XCHECK HALTED DUE TO IMPROPER BREAKPOINTING

OR EXECUTION FOLLOWING "STEP" COMMAND.

is written, and control is passed to the idbg16 command handler.

If the Compare Breakpoint Status code is not h'15', the message:

XCHECK HALTED DUE TO IMPROPER COMPARE-BREAKPOINT STATUS

is written, and control is passed to the idbg16 command handler.

If the Trace Buffer Address Counter is not equal to one, the

message:

XCHECK HALTED DUE TO IMPROPER TRACE BUFFER-ADDRESS VALUE

is written, and control is passed to the idbg16 command handler.

If the content of Trace RAM 0 (low data) is not correct, the

message:

XCHECK HALTED DUE TO INCORRECT LOW-DATA TRACE

is written, and control is passed to the idbg16 command handler.

If the content of Trace RAM 1 (high data) is not correct, the

message:

XCHECK HALTED DUE TO INCORRECT HIGH-DATA TRACE

is written, and control is passed to the idbg16 command handler.

If the content of Trace RAM 2 (low address) is not correct, the

message:

XCHECK HALTED DUE TO INCORRECT LOW-ADDRESS TRACE

is written, and control is passed to the idbg16 command handler.

If the content of Trace RAM 3 (middle address) is not correct,

the message:

XCHECK HALTED DUE TO INCORRECT MIDDLE-ADDRESS TRACE

is written, and control is passed to the idbg16 command handler.

If the content of Trace RAM 4 (high address) is not correct, the

message:

XCHECK HALTED DUE TO INCORRECT HIGH-ADDRESS TRACE

is written, and control is passed to the idbg16 command handler.

If the content of Trace RAM 5 (status bits) is not correct, the

message:

XCHECK HALTED DUE TO INCORRECT STATUS-BIT TRACE

is writte, and control is passed to the idbg16 command handler.

18)  cbrk20.ind

Description:  This file checks breakpoint and trace for an input

pattern in which zeros are applied to all bits except bit 5 of

each of the Compare Breakpoint RAMs.  This is accomplished by

single-stepping an instruction at the address which satisfies

the bit 5 requirement, and breakpointing and tracing the execution

of that instruction.

Detectable failures:  If the PC is incorrect following the

emulation, the message:

XCHECK HALTED DUE TO IMPROPER BREAKPOINTING

OR EXECUTION FOLLOWING "STEP" COMMAND.

is written, and control is passed to the idbg16 command handler.

If the Compare Breakpoint Status code is not h'15', the message:

XCHECK HALTED DUE TO IMPROPER COMPARE-BREAKPOINT STATUS

is written, and control is passed to the idbg16 command handler.

If the Trace Buffer Address Counter is not equal to one, the

message:

XCHECK HALTED DUE TO IMPROPER TRACE BUFFER-ADDRESS VALUE

is written, and control is passed to the idbg16 command handler.

If the content of Trace RAM 0 (low data) is not correct, the

message:

XCHECK HALTED DUE TO INCORRECT LOW-DATA TRACE

is written, and control is passed to the idbg16 command handler.

If the content of Trace RAM 1 (high data) is not correct, the

message:

XCHECK HALTED DUE TO INCORRECT HIGH-DATA TRACE

is written, and control is passed to the idbg16 command handler.

If the content of Trace RAM 2 (low address) is not correct, the

message:

XCHECK HALTED DUE TO INCORRECT LOW-ADDRESS TRACE

is written, and control is passed to the idbg16 command handler.

If the content of Trace RAM 3 (middle address) is not correct,

the message:

XCHECK HALTED DUE TO INCORRECT MIDDLE-ADDRESS TRACE

is written, and control is passed to the idbg16 command handler.

If the content of Trace RAM 4 (high address) is not correct, the

message:

XCHECK HALTED DUE TO INCORRECT HIGH-ADDRESS TRACE

is written, and control is passed to the idbg16 command handler.

If the content of Trace RAM 5 (status bits) is not correct, the

message:

XCHECK HALTED DUE TO INCORRECT STATUS-BIT TRACE

is written, and control is passed to the idbg16 command handler.

19)  cbrk40.ind

Description:  This file checks breakpoint and trace for an input

pattern in which zeros are applied to all bits except bit 6 of

each of the Compare Breakpoint RAMs.  This is accomplished by

single-stepping an instruction at the address which satisfies

the bit 6 requirement, and breakpointing and tracing the execution

of that instruction.

Detectable failures:  If the PC is incorrect following the

emulation, the message:

XCHECK HALTED DUE TO IMPROPER BREAKPOINTING

OR EXECUTION FOLLOWING "STEP" COMMAND.

is written, and control is passed to the idbg16 command handler.

If the Compare Breakpoint Status code is not h'15', the message:

XCHECK HALTED DUE TO IMPROPER COMPARE-BREAKPOINT STATUS

is written, and control is passed to the idbg16 command handler.

If the Trace Buffer Address Counter is not equal to one, the

message:

XCHECK HALTED DUE TO IMPROPER TRACE BUFFER-ADDRESS VALUE

is written, and control is passed to the idbg16 command handler.

If the content of Trace RAM 0 (low data) is not correct, the

message:

XCHECK HALTED DUE TO INCORRECT LOW-DATA TRACE

is written, and control is passed to the idbg16 command handler.

If the content of Trace RAM 1 (high data) is not correct, the

message:

XCHECK HALTED DUE TO INCORRECT HIGH-DATA TRACE

is written, and control is passed to the idbg16 command handler.

If the content of Trace RAM 2 (low address) is not correct, the

message:

XCHECK HALTED DUE TO INCORRECT LOW-ADDRESS TRACE

is written, and control is passed to the idbg16 command handler.

If the content of Trace RAM 3 (middle address) is not correct,

the message:

XCHECK HALTED DUE TO INCORRECT MIDDLE-ADDRESS TRACE

is written, and control is passed to the idbg16 command handler.

If the content of Trace RAM 4 (high address) is not correct, the

message:

XCHECK HALTED DUE TO INCORRECT HIGH-ADDRESS TRACE

is written, and control is passed to the idbg16 command handler.

If the content of Trace RAM 5 (status bits) is not correct, the

message:

XCHECK HALTED DUE TO INCORRECT STATUS-BIT TRACE

is written, and control is passed to the idbg16 command handler.

20)  cbrk80.ind

Description:  This file checks breakpoint and trace for an input

pattern in which zeros are applied to all bits except bit 7 of

each of the Compare Breakpoint RAMs.  This is accomplished by

single-stepping an instruction at the address which satisfies

the bit 7 requirement, and breakpointing and tracing the execution

of that instruction.

Detectable failures:  If the PC is incorrect following the

emulation, the message:

XCHECK HALTED DUE TO IMPROPER BREAKPOINTING

OR EXECUTION FOLLOWING "STEP" COMMAND.

is written, and control is passed to the idbg16 command handler.

If the Compare Breakpoint Status code is not h'15', the message:

XCHECK HALTED DUE TO IMPROPER COMPARE-BREAKPOINT STATUS

is written, and control is passed to the idbg16 command handler.

If the Trace Buffer Address Counter is not equal to one, the

message:

XCHECK HALTED DUE TO IMPROPER TRACE BUFFER-ADDRESS VALUE

is written, and control is passed to the idbg16 command handler.

If the content of Trace RAM 0 (low data) is not correct, the

message:

XCHECK HALTED DUE TO INCORRECT LOW-DATA TRACE

is written, and control is passed to the idbg16 command handler.

If the content of Trace RAM 1 (high data) is not correct, the

message:

XCHECK HALTED DUE TO INCORRECT HIGH-DATA TRACE

is written, and control is passed to the idbg16 command handler.

If the content of Trace RAM 2 (low address) is not correct, the

message:

XCHECK HALTED DUE TO INCORRECT LOW-ADDRESS TRACE

is written, and control is passed to the idbg16 command handler.

If the content of Trace RAM 3 (middle address) is not correct,

the message:

XCHECK HALTED DUE TO INCORRECT MIDDLE-ADDRESS TRACE

is written, and control is passed to the idbg16 command handler.

If the content of Trace RAM 4 (high address) is not correct, the

message:

XCHECK HALTED DUE TO INCORRECT HIGH-ADDRESS TRACE

is written, and control is passed to the idbg16 command handler.

If the content of Trace RAM 5 (status bits) is not correct, the

message:

XCHECK HALTED DUE TO INCORRECT STATUS-BIT TRACE

is written, and control is passed to the idbg16 command handler.

21)  cdst01.ind

Description:  This file checks breakpointing for an input pattern

in which zeros are applied to all bits except bit 0 of the

Compare Distribution Breakpoint RAM.  This is accomplished by

executing a program in Emulation RAM with the breakpoints set

appropriatly.

Detectable failures:  If the content of the PC is not correct,

the message:

XCHECK HALTED DUE TO IMPROPER BREAKPOINTING

OR EXECUTION FOLLOWING "GO" COMMAND

is written, and control is passed to the idbg16 command handler.

If the value of the Compare Breakpoint Status code is incorrect,

the message:

XCHECK HALTED DUE TO IMPROPER COMPARE-BREAKPOINT STATUS

is written, and control is passed to the idbg16 command handler.

If the value of r0 or r1 is incorrect, improper program execution

is indicated (probably due to breakpointing error), and the

message:

XCHECK HALTED DUE TO IMPROPER PROGRAM EXECUTION

is written, and control is passed to the idbg16 command handler.

22)  cdst02.ind

Description:  This file checks breakpointing for an input pattern

in which zeros are applied to all bits except bit 1 of the

Compare Distribution Breakpoint RAM.  This is accomplished by

executing a program in Emulation RAM with the breakpoints set

appropriatly.

Detectable failures:  If the content of the PC is not correct,

the message:

XCHECK HALTED DUE TO IMPROPER BREAKPOINTING

OR EXECUTION FOLLOWING "GO" COMMAND

is written, and control is passed to the idbg16 command handler.

If the value of the Compare Breakpoint Status code is incorrect,

the message:

XCHECK HALTED DUE TO IMPROPER COMPARE-BREAKPOINT STATUS

is written, and control is passed to the idbg16 command handler.

If the value of r0 or r1 is incorrect, improper program execution

is indicated (probably due to breakpointing error), and the

message:

XCHECK HALTED DUE TO IMPROPER PROGRAM EXECUTION

is written, and control is passed to the idbg16 command handler.

23)  cdst04.ind

Description:  This file checks breakpointing for an input pattern

in which zeros are applied to all bits except bit 2 of the

Compare Distribution Breakpoint RAM.  This is accomplished by

executing a program in Emulation RAM with the breakpoints set

appropriatly.

Detectable failures:  If the content of the PC is not correct,

the message:

XCHECK HALTED DUE TO IMPROPER BREAKPOINTING

OR EXECUTION FOLLOWING "GO" COMMAND

is written, and control is passed to the idbg16 command handler.

If the value of the Compare Breakpoint Status code is incorrect,

the message:

XCHECK HALTED DUE TO IMPROPER COMPARE-BREAKPOINT STATUS

is written, and control is passed to the idbg16 command handler.

If the value of r0 or r1 is incorrect, improper program execution

is indicated (probably due to breakpointing error), and the

message:

XCHECK HALTED DUE TO IMPROPER PROGRAM EXECUTION

is written, and control is passed to the idbg16 command handler.

24)  cdst08.ind

Description:  This file checks breakpointing for an input pattern

in which zeros are applied to all bits except bit 3 of the

Compare Distribution Breakpoint RAM.  This is accomplished by

executing a program in Emulation RAM with the breakpoints set

appropriatly.

Detectable failures:  If the content of the PC is not correct,

the message:

XCHECK HALTED DUE TO IMPROPER BREAKPOINTING

OR EXECUTION FOLLOWING "GO" COMMAND

is written, and control is passed to the idbg16 command handler.

If the value of the Compare Breakpoint Status code is incorrect,

the message:

XCHECK HALTED DUE TO IMPROPER COMPARE-BREAKPOINT STATUS

is written, and control is passed to the idbg16 command handler.

If the value of r0 or r1 is incorrect, improper program execution

is indicated (probably due to breakpointing error), and the

message:

XCHECK HALTED DUE TO IMPROPER PROGRAM EXECUTION

is written, and control is passed to the idbg16 command handler.

25)  cdst10.ind

Description:  This file checks breakpointing for an input pattern

in which zeros are applied to all bits except bit 4 of the

Compare Distribution Breakpoint RAM.  This is accomplished by

executing a program in Emulation RAM with the breakpoints set

appropriatly.

Detectable failures:  If the content of the PC is not correct,

the message:

XCHECK HALTED DUE TO IMPROPER BREAKPOINTING

OR EXECUTION FOLLOWING "GO" COMMAND

is written, and control is passed to the idbg16 command handler.

If the value of the Compare Breakpoint Status code is incorrect,

the message:

XCHECK HALTED DUE TO IMPROPER COMPARE-BREAKPOINT STATUS

is written, and control is passed to the idbg16 command handler.

If the value of r0 or r1 is incorrect, improper program execution

is indicated (probably due to breakpointing error), and the

message:

XCHECK HALTED DUE TO IMPROPER PROGRAM EXECUTION

is written, and control is passed to the idbg16 command handler.

26)  cdst20.ind

Description:  This file checks breakpointing for an input pattern

in which zeros are applied to all bits except bit 5 of the

Compare Distribution Breakpoint RAM.  This is accomplished by

executing a program in Emulation RAM with the breakpoints set

appropriatly.

Detectable failures:  If the content of the PC is not correct,

the message:

XCHECK HALTED DUE TO IMPROPER BREAKPOINTING

OR EXECUTION FOLLOWING "GO" COMMAND

is written, and control is passed to the idbg16 command handler.

If the value of the Compare Breakpoint Status code is incorrect,

the message:

XCHECK HALTED DUE TO IMPROPER COMPARE-BREAKPOINT STATUS

is written, and control is passed to the idbg16 command handler.

If the value of r0 or r1 is incorrect, improper program execution

is indicated (probably due to breakpointing error), and the

message:

XCHECK HALTED DUE TO IMPROPER PROGRAM EXECUTION

is written, and control is passed to the idbg16 command handler.

27)  cdst40.ind

Description:  This file checks breakpointing for an input pattern

in which zeros are applied to all bits except bit 6 of the

Compare Distribution Breakpoint RAM.  This is accomplished by

executing a program in Emulation RAM with the breakpoints set

appropriatly.

Detectable failures:  If the content of the PC is not correct,

the message:

XCHECK HALTED DUE TO IMPROPER BREAKPOINTING

OR EXECUTION FOLLOWING "GO" COMMAND

is written, and control is passed to the idbg16 command handler.

If the value of the Compare Breakpoint Status code is incorrect,

the message:

XCHECK HALTED DUE TO IMPROPER COMPARE-BREAKPOINT STATUS

is written, and control is passed to the idbg16 command handler.

If the value of r0 or r1 is incorrect, improper program execution

is indicated (probably due to breakpointing error), and the

message:

XCHECK HALTED DUE TO IMPROPER PROGRAM EXECUTION

is written, and control is passed to the idbg16 command handler.

28)  cdst80.ind

Description:  This file checks breakpointing for an input pattern

in which zeros are applied to all bits except bit 7 of the

Compare Distribution Breakpoint RAM.  This is accomplished by

executing a program in Emulation RAM with the breakpoints set

appropriatly.

Detectable failures:  If the content of the PC is not correct,

the message:

XCHECK HALTED DUE TO IMPROPER BREAKPOINTING

OR EXECUTION FOLLOWING "GO" COMMAND

is written, and control is passed to the idbg16 command handler.

If the value of the Compare Breakpoint Status code is incorrect,

the message:

XCHECK HALTED DUE TO IMPROPER COMPARE-BREAKPOINT STATUS

is written, and control is passed to the idbg16 command handler.

If the value of r0 or r1 is incorrect, improper program execution

is indicated (probably due to breakpointing error), and the

message:

XCHECK HALTED DUE TO IMPROPER PROGRAM EXECUTION

is written, and control is passed to the idbg16 command handler.

29)  trats1.ind

Description:  This file checks the function of the Trace Buffer

Address Counter for unconditional Memory-Cycle Trace.

Detectable failures:  If the Trace Buffer Address Counter or

Counter Overflow bit are not correct, the message:

XCHECK HALTED DUE TO IMPROPER TRACE BUFFER-ADDRESS VALUE

is written, and control is passed to the idbg16 command handler.

30)  trats2.ind

Description:  This file checks the function of the Trace Coast

Counter for unconditional Memory-Cycle Trace, with pre-trigger

specified by the define-trace command.

Detectable failures:  If the Trace Buffer Address Counter or

Counter Overflow bit are not correct, the message:

XCHECK HALTED DUE TO IMPROPER TRACE BUFFER-ADDRESS VALUE

is written, and control is passed to the idbg16 command handler.

If the Trace Coast Counter Done bit is not set, the message:

XCHECK HALTED DUE TO IMPROPER TRACE COAST-COUNTER ACTION

is written, and control is passed to the idbg16 command handler.

31)  trats3.ind

Description:  This file checks the function of the Trace Coast

Counter for unconditional Memory-Cycle Trace, with center-trigger

specified by the define-trace command.

Detectable failures:  If the Trace Buffer Address Counter or

Counter Overflow bit are not correct, the message:

XCHECK HALTED DUE TO IMPROPER TRACE BUFFER-ADDRESS VALUE

is written, and control is passed to the idbg16 command handler.

If the Trace Coast Counter Done bit is not set, the message:

XCHECK HALTED DUE TO IMPROPER TRACE COAST-COUNTER ACTION

is written, and control is passed to the idbg16 command handler.

32)  trats4.ind

Description:  This file checks the function of the Trace Coast

Counter for unconditional Memory-Cycle Trace, with post-trigger

specified by the define-trace command.

Detectable failures:  If the Trace Buffer Address Counter or

Counter Overflow bit are not correct, the message:

XCHECK HALTED DUE TO IMPROPER TRACE BUFFER-ADDRESS VALUE

is written, and control is passed to the idbg16 command handler.

If the Trace Coast Counter Done bit is not set, the message:

XCHECK HALTED DUE TO IMPROPER TRACE COAST-COUNTER ACTION

is written, and control is passed to the idbg16 command handler.

33)  trats5.ind

Description:  This file checks the operation of the NPFS-count

recorder of the Memory-Cycle Trace logic.

Detectable failures:  If the content of the NPFS-count RAM is

not correct, the message:

XCHECK HALTED DUE TO INCORRECT TRACE PROGRAM-FLOW STROBE COUNT

is written, and control is passed to the idbg16 command handler.

34)  trats6.ind

Description:  This file checks the operation of Program-Flow

Trace.

Detectable failures:  If the Trace Buffer Address Counter has

an incorrect value, the messsage:

XCHECK HALTED DUE TO IMPROPER TRACE BUFFER-ADDRESS VALUE

is written, and control is passed to the idbg16 command handler.

If Trace RAM 0 (low byte of count) does not have the proper

value, the message:

XCHECK HALTED DUE TO INCORRECT LOW-DATA TRACE

is written, and control is passed to the idbg16 command handler.

If Trace RAM 1 (high byte of count) does not have the proper

value, the message:

XCHECK HALTED DUE TO INCORRECT HIGH-DATA TRACE

is written, and control is passed to the idbg16 command handler.

If Trace RAM 2 (low address) does not have the proper value, the

message:

XCHECK HALTED DUE TO INCORRECT LOW-ADDRESS TRACE

is written, and control is passed to the idbg16 command handler.

If Trace RAM 3 (middle address) does not have the proper value,

the message:

XCHECK HALTED DUE TO INCORRECT MIDDLE-ADDRESS TRACE

is written, and control is passed to the idbg16 command handler.

If Trace RAM 4 (high address) does not have the proper value,

the message:

XCHECK HALTED DUE TO INCORRECT HIGH-ADDRESS TRACE

is written, and control is passed to the idbg16 command handler.

If Trace RAM 5 (status bits) does not have the proper value, the

message:

XCHECK HALTED DUE TO INCORRECT STATUS-BIT TRACE

is written, and control is passed to the idbg16 command handler.

35)  trats7.ind

Description:  This file checks the operation of triggered

Program-Flow Trace.

Detectable failures:  If the Trace Buffer Address Counter has

an incorrect value, the messsage:

XCHECK HALTED DUE TO IMPROPER TRACE BUFFER-ADDRESS VALUE

is written, and control is passed to the idbg16 command handler.

If Trace RAM 0 (low byte of count) does not have the proper

value, the message:

XCHECK HALTED DUE TO INCORRECT LOW-DATA TRACE

is written, and control is passed to the idbg16 command handler.

If Trace RAM 1 (high byte of count) does not have the proper

value, the message:

XCHECK HALTED DUE TO INCORRECT HIGH-DATA TRACE

is written, and control is passed to the idbg16 command handler.

If Trace RAM 2 (low address) does not have the proper value, the

message:

XCHECK HALTED DUE TO INCORRECT LOW-ADDRESS TRACE

is written, and control is passed to the idbg16 command handler.

If Trace RAM 3 (middle address) does not have the proper value,

the message:

XCHECK HALTED DUE TO INCORRECT MIDDLE-ADDRESS TRACE

is written, and control is passed to the idbg16 command handler.

If Trace RAM 4 (high address) does not have the proper value,

the message:

XCHECK HALTED DUE TO INCORRECT HIGH-ADDRESS TRACE

is written, and control is passed to the idbg16 command handler.

If Trace RAM 5 (status bits) does not have the proper value, the

message:

XCHECK HALTED DUE TO INCORRECT STATUS-BIT TRACE

is written, and control is passed to the idbg16 command handler.

36)  imbrk1.ind

Description:  This file checks the action of the Break-Counter

Done flag in creating an end-of-run breakpoint.

Detectable failures:  If the value of the PC is not correct, the

message:

XCHECK HALTED DUE TO IMPROPER BREAKPOINTING

OR EXECUTION FOLLOWING "GO" COMMAND

is written, and control is passed to the idbg16 command handler.

If the Compare Breakpoint Status code is not correct, the message:

XCHECK HALTED DUE TO IMPROPER IMMEDIATE-BREAKPOINT STATUS

is written, and control is passed to the idbg16 command handler.

If the Immediate Breakpoint Status code is not correct, the

message:

XCHECK HALTED DUE TO IMPROPER IMMEDIATE-BREAKPOINT STATUS

is written, and control is passed to the idbg16 command handler.

37)  imbrk2.ind

Description:  This file checks the action of the Trace Coast

Counter Done flag in creating an end-of-run breakpoint.

Detectable failures:  If the value of the PC is not correct, the

message:

XCHECK HALTED DUE TO IMPROPER BREAKPOINTING

OR EXECUTION FOLLOWING "GO" COMMAND

is written, and control is passed to the idbg16 command handler.

If the Compare Breakpoint Status code is not correct, the message:

XCHECK HALTED DUE TO IMPROPER IMMEDIATE-BREAKPOINT STATUS

is written, and control is passed to the idbg16 command handler.

If the Immediate Breakpoint Status code is not correct, the

message:

XCHECK HALTED DUE TO IMPROPER IMMEDIATE-BREAKPOINT STATUS

is written, and control is passed to the idbg16 command handler.

38)  imbrk4.ind

Description:  This file checks the action of Illegal Map flag in

creating an end-of-run breakpoint, and verifies that write-

protected Emulation RAM locations are not modified on a data-

write.

Detectable failures:  If the value of the PC is not correct, the

message:

XCHECK HALTED DUE TO IMPROPER BREAKPOINTING

OR EXECUTION FOLLOWING "GO" COMMAND

is written, and control is passed to the idbg16 command handler.

If the Compare Breakpoint Status code is not correct, the message:

XCHECK HALTED DUE TO IMPROPER IMMEDIATE-BREAKPOINT STATUS

is written, and control is passed to the idbg16 command handler.

If the Immediate Breakpoint Status code is not correct, or the

write-protected memory location has been modified, the message:

XCHECK HALTED DUE TO IMPROPER IMMEDIATE-BREAKPOINT STATUS

is written, and control is passed to the idbg16 command handler.

39)  extmrc.ind

Description:  This file checks the operation of the Execution

Timer for the CTTL input.  It checks incremental values as each

counter in the hardware assumes a value, and checks the final

value at the end of the test.

Detectable failures:  If the value of the PC is incorrect, the

message:

XCHECK HALTED DUE TO IMPROPER BREAKPOINTING

OR EXECUTION FOLLOWING "GO" COMMAND

is written, and control is passed to the idbg16 command handler.

If the value of any counter is incorrect at any time, the message:

XCHECK HALTED DUE TO INCORRECT EXECUTION TIMER VALUE

is written, and control is passed to the idbg16 command handler.

40)  extmrm.ind

Description:  This file checks the operation of the Execution

Timer for the NADS input.  It checks the final counter value at

the end of the test.

Detectable failures:  If the value of the PC is incorrect, the

message:

XCHECK HALTED DUE TO IMPROPER BREAKPOINTING

OR EXECUTION FOLLOWING "GO" COMMAND

is written, and control is passed to the idbg16 command handler.

If the value of any counter is incorrect at any time, the message:

XCHECK HALTED DUE TO INCORRECT EXECUTION TIMER VALUE

is written, and control is passed to the idbg16 command handler.

41)  extmri.ind

Description:  This file checks the operation of the Execution

Timer for the NPFS input.  It checks the final counter value at

the end of the test.

Detectable failures:  If the value of the PC is incorrect, the

message:

XCHECK HALTED DUE TO IMPROPER BREAKPOINTING

OR EXECUTION FOLLOWING "GO" COMMAND

is written, and control is passed to the idbg16 command handler.

If the value of any counter is incorrect at any time, the message:

XCHECK HALTED DUE TO INCORRECT EXECUTION TIMER VALUE

is written, and control is passed to the idbg16 command handler.

42)  extmrb.ind

Description:  This file checks the operation of the Execution

Timer for the Breakpoint input.  It checks the final counter value

at the end of the test.

Detectable failures:  If the value of any counter is incorrect

at any time, the message:

XCHECK HALTED DUE TO INCORRECT EXECUTION TIMER VALUE

is written, and control is passed to the idbg16 command handler.

43)  bkctrc.ind

Description:  This file uses the Execution Timer to verify proper

counting operation of the Breakpoint Counter for the CTTL input.

Detectable failures:  If the value of the Compare Breakpoint

Status code is incorrect, the message:

XCHECK HALTED DUE TO IMPROPER COMPARE-BREAKPOINT STATUS

is written, and control is passed to the idbg16 command handler.

If the value of the Execution Timer does not correspond to the

value written to the Breakpoint Counter, the message:

XCHECK HALTED DUE TO INCORRECT EXECUTION TIMER VALUE

is written, and control is passed to the idbg16 command handler.

44)  bkctrm.ind

Description:  This file uses the Execution Timer to verify proper

counting operation of the Breakpoint Counter for the NADS input.

Detectable failures:  If the value of the Compare Breakpoint

Status code is incorrect, the message:

XCHECK HALTED DUE TO IMPROPER COMPARE-BREAKPOINT STATUS

is written, and control is passed to the idbg16 command handler.

If the value of the Execution Timer does not correspond to the

value written to the Breakpoint Counter, the message:

XCHECK HALTED DUE TO INCORRECT EXECUTION TIMER VALUE

is written, and control is passed to the idbg16 command handler.

45)  bkctri.ind

Description:  This file uses the Execution Timer to verify proper

counting operation of the Breakpoint Counter for the NPFS input.

Detectable failures:  If the value of the Compare Breakpoint

Status code is incorrect, the message:

XCHECK HALTED DUE TO IMPROPER COMPARE-BREAKPOINT STATUS

is written, and control is passed to the idbg16 command handler.

If the value of the Execution Timer does not correspond to the

value written to the Breakpoint Counter, the message:

XCHECK HALTED DUE TO INCORRECT EXECUTION TIMER VALUE

is written, and control is passed to the idbg16 command handler.

46)  bkctrb.ind

Description:  This file uses the Execution Timer to verify proper

counting operation of the Breakpoint Counter for the Breakpoint

input.

Detectable failures:  If the value of the Compare Breakpoint

Status code is incorrect, the message:

XCHECK HALTED DUE TO IMPROPER COMPARE-BREAKPOINT STATUS

is written, and control is passed to the idbg16 command handler.

If the value of the Execution Timer does not correspond to the

value written to the Breakpoint Counter, the message:

XCHECK HALTED DUE TO INCORRECT EXECUTION TIMER VALUE

is written, and control is passed to the idbg16 command handler.